Business Blueprint AOP Automation FOR MEG PlanT

| Site Name | Reliance Industries Ltd. |
| --- | --- |
| Track Name | RIL AOP Automation for MEG Plant |
| Rev. No. | Rev 5 |
| Rev. Date | 02-06-2025 |
| Honeywell Doc No. |  |
| RIL Doc No. |  |

Business Blueprint

# AOP Automation for MEG Plant

Prepared for:

Reliance Industries Ltd

Submitted by:

Honeywell Automation India Ltd (“Honeywell”)

# Revision History

| Rev. | Date | Originated By | Description of status and changes |
| --- | --- | --- | --- |
| 0.0 | 12-Nov-2024 | HON | Issued for Review |
| 1.0 | 30-Jan-2025 | HON | Issued for Review |
| 2.0 | 06-Feb-2025 | HON | Issued for Review |
| 3.0 | 14-Feb-2025 | HON | Issued for Revied (RIL comments included) |
| 5.0 | 02-June-2025 | HON | Issued for Approval |

Table of Contents

1.	Introduction	6

1.1	Executive Summary	6

1.2	Reference Documents	7

2.	Workshop Details	8

3.	AOP Design Basis	9

3.1	Process and Operations Function	9

3.1.1	Budget Production/Production AOP	10

3.1.2	Norms	17

3.1.3	Feedstock Availability	23

3.1.4	Other Cost (To generate cost sheet - Business)	23

4.	AOP Solution Overview/Output	24

5.	Key Assumptions and Exemptions	25

# List of Figures

Figure 1 Digitized AOP Flow Diagram	9

Figure 2 Digitized AOP screen for Configuration/manual Inputs	10

Figure 3 Digitized AOP Steps to derive EOE and EO month wise MC values	11

Figure 4 Digitized AOP screen for MEG Yield	11

Figure 5 Digitized AOP screen for Production Volume data	12

Figure 6 Digitized AOP screen for Business Demand	12

Figure 7 Digitized AOP screen for Shutdown/Turnaround Activities	13

Figure 8 Digitized AOP screen for Slowdown Activities	13

Figure 9 AOP Maintenance Details Table	15

Figure 10 Digitized AOP screen for Production AOP month wise	16

Figure 11 Digitized AOP Steps to derive month wise Historical Best Norms	17

Figure 12 Example of selectivity Extrapolation	18

Figure 13 Digitized AOP screen for Catalyst Selectivity month wise	19

Figure 14 Digitized AOP screen for Normal operations Norms	19

Figure 15 Digitized AOP screen for Shutdown Norms	21

Figure 16 Digitized AOP screen for Shutdown Norms	21

Figure 17 Digitized AOP screen for Budget Norms or Consumption Norms	22

Figure 18 Digitized AOP screen for Feed Stock Availability	23

# List of Tables

Table 1 Reference Documents	7

Table 2 List of MEG vertical Plants	8

Table 3 AOP Functional Scope	9

Table 4 Maintenance Details	14

Table 5 Raw Material Norms calculations	18

# Introduction

The purpose of this document is to capture the detailed specifications for the Digitalized Annual Operating Plan (AOP) for MEG technology plants across RIL sites.

This document is based on the Technical Proposal submitted by Honeywell to Reliance. It also refers to data collected during the workshop conducted at RCP with various MEG plant users across RIL sites.

## Executive Summary

Reliance Industries Limited is India’s leading private sector organization operating in various business verticals. The manufacturing sector is one such business vertical. RIL owns and operates multiple plants across various sites.

Defining and finalizing the Annual Operating Plan is a critical annual business process carried out by different functional teams. At present, this activity is carried out manually.

A Rev-0 budget is usually completed in Nov / Dec for the succeeding Financial Year (FY) and reviewed. The comments are addressed with updated information from the Business, as the next revised budget is expected by the end of January. Further revisions (if any) are incorporated in Feb / March to reflect updated business input as applicable.

The entries from the final rev of the budget are finalized in BPC before the beginning of the succeeding FY.

RIL aims to automate the AOP preparation process using a Data-Driven Model. This model leverages the last five years of AOP data from MIIS and incorporates plant-specific business rules (such as plant availability, running hours, feedstock availability, seasonal impact on production, etc.) to derive AOP figures in the expected formats.

As part of digitalization, Honeywell will support building a platform that will automate the Annual Operations Planning process for MEG technology.

## Reference Documents

| Document ID | Revision | Title | Document |
| --- | --- | --- | --- |
| O-836243B24R01 | R01 | AOP Automation for O2C Proposal |  |
| Workshop MOM | - | MEG_Norms_Steps |  |
| MEG DCT | - | DCT for EOEG Digital AOP |  |

Table 1 Reference Documents

# Workshop Details

HON conducted a workshop along with the RIL MEG technology teams dated 7th and 8th Oct 2024.

During this workshop, current practices followed for the development of an AOP plan for the process and operation function, of respective sites/plants for a financial year were identified.

The summary of details discussed with the RIL team is captured in the Excel (MEG_Norms_Steps) provided in the reference document table.

The MEG vertical consists of the below plants for digitized AOP preparation

| Sr. No. | Sites for MEG | Plants | Workshop Participants |
| --- | --- | --- | --- |
| 1 | C2 | MEG | Site/COE Team |
| 2 | DMD | EOEG | Site/COE Team |
| 3 | HMD | MEG 1 | Site/COE Team |
| 4 | HMD | MEG 2 | Site/COE Team |
| 5 | HMD | MEG 3 | Site/COE Team |
| 6 | NMD | EOEG | Site/COE Team |
| 7 | VMD | EOEG | Site/COE Team |

Table 2 List of MEG vertical Plants

# AOP Design Basis

The table below captures different functions for the scope of AOP digitization design –

| Sr. No. | Functions | Covered in Document |
| --- | --- | --- |
| 1. | Health, Safety and Environment (HSE) | For later release, post review and discussion |
| 2. | Process/Operations | Covered |
| 3. | Reliability and Maintenance | For later release, post review and discussion |

Table 3 AOP Functional Scope

Function-wise, the detailed design basis is captured in the sections below.

## Process and Operations Function

In this section, the budgeted Production and Norms for a plant are derived, using a standard approach as briefed in the sections below for the following criteria

- Budget Production/Production AOP

- Norms/Consumption AOP

Steps to generate the Production and Consumption AOP are captured in the below flow diagram.

Figure 1 Digitized AOP Flow Diagram

### Budget Production/Production AOP

Below are the steps captured to derive the budgeted AOP production values.

#### Configuration

The configuration screen serves as the foundation for the digital AOP process, allowing users to input essential data for production and consumption AOP calculations.

Plant-specific data, including Yield, Selectivity, and Consumption, are configured under the Configuration screen to support Production and Consumption Norms calculations and reports generation.

By default, the values entered for the current year will be automatically carried forward to the next year.

The user must insert the TST value month wise and save it with a valid reason. This will provide the Latent heat against TST.

Note - Latent heat will be calculated only if the user provides the correct pressure input corresponding to the TST temperature; otherwise, the latent heat value will be set to 0.

The screenshot below highlights the details captured in the digitized AOP platform–

Figure 2 Digitized AOP screen for Configuration/manual Inputs

#### Production Volume Data

The Max Capacity (MC) of EOEG plants is defined on an EOE basis.

MC values for Ethylene Oxide (EO), Ethylene Oxide Equivalent (EOE) and Monoethylene Glycol (MEG) will be captured separately.

Month wise data is considered for deriving the best value. This will account for seasonal effects as well as the festive season.

Value estimation involves averaging production data over continuous 5-day periods in the last 5 years, after removing outliers. For each month, 5 such averages are calculated, and the maximum value from the highest average set is designated as the MC value or production volume.

A similar approach is employed for calculating the EO MC value.

The following logic has been formulated to derive month-wise EOE and EO MC values.

Figure 3 Digitized AOP Steps to derive EOE and EO month wise MC values

The MEG MC value is determined using the following formula.

MEG MC = MEG Yield*(EOE MC-EO MC)

Where MEG yield is a month-wise manual entry captured in the AOP digitization portal in the Configuration screen, as seen in the screenshot below.

Figure 4 Digitized AOP screen for MEG Yield

If the user needs to modify the production rate this can be done by providing a specific reason.

The AOP digitization portal captures a month-wise percentage summary, comparing each month's value against the highest MU value recorded over the past 12 months.

This information is captured in the AOP digitization portal as seen in the screenshot below.

Figure 5 Digitized AOP screen for Production Volume data

The 'Calculate' button on this screen allows the user to compute the MEG MC value if any changes have been made to the MEG Yield on the configuration screen.

#### Business Demand

The production demand is considered as the maximum value from last year's achieved production values. The default value for EOE, EO and MEG is 100%.

The business demand value can be modified by users upon providing a specific reason.

This information is captured in the AOP digitization portal as seen in the screenshot below.

NOTE: For the HMD MEG1, NMD and VMD plants, the CO2 demand will be captured in this section.

Figure 6 Digitized AOP screen for Business Demand

#### Maintenance/Shutdown/Slowdown/TA Activities

The operations team will provide the shutdown, turnaround and slowdown plan to calculate month-wise operating hours available for production.

The below screenshot depicts the shutdown and turnaround activities planned, as will be captured in the AOP digitized platform.

The user has to enter the SD-From and SD-To date and time to calculate the duration in hours, or SD-From and Duration(hrs) to calculate the SD-To date/time.

Figure 7 Digitized AOP screen for Shutdown/Turnaround Activities

The Slowdown entries will have the inputs as shutdowns, along with the requirement of plant EOE/EO load (TPH basis) for the duration of the slowdown.

The slowdown activities can be calculated based on the slowdown rate and hours. This is called a Basis Shutdown, equivalent to Shutdown.

Basis shutdown equivalent of Slowdown hrs = (Slowdown rate(TPH) * Slowdown hrs) / MCU

For each shutdown entry, four slowdown entries will be generated automatically, with the nomenclature as “#Shutdown Entry name. – Ramp down” and “#Shutdown Entry name. – Ramp Up”, two for each EO and EOE.

The below screen depicts the slowdown activities planned which is captured in the digitized platform.

Figure 8 Digitized AOP screen for Slowdown Activities

The 'Add Item' button on the screens allows users to insert a new entry for shutdown, turnaround, or slowdown.

The 'Save' button enables users to save newly added items as well as any modifications made to existing entries.

Using this data, the number of operating hours available in a month is calculated as -

Number of Operating Hours in a month = Total Number of hrs in a month – (Shutdown Hrs + Slowdown converted to equivalent Shutdown Hrs)

For each month, the following table is generated based on all inputs for shutdowns/slowdowns-

| Description | April | … | March |
| --- | --- | --- | --- |
| Total Available Hours for the Month | 30*24=720 |  |  |
| Shutdowns Hours | From the Routine Shutdown Plan Section |  |  |
| Non-Shutdown Operating Hours for the Month | Total Available operating hours for the month-Shutdown hrs |  |  |
| Average Slowdown Load/rate for EOE, % of Production Volume Target (PVT) | i=1n(EOE slowdown hrsi×EOE load/rate during slowdowni)i=1n(EOE slowdown hrsi) ×EOE MC for Month |  |  |
| Average Slowdown Load/rate for EO, % of Production Volume Target (PVT) | i=1n(EO slowdown hrsi×EO load/rate during slowdowni)i=1n(EO slowdown hrsi) ×EO MC for Month |  |  |
| EOE Slowdown Load/rate Reduction as Shutdown Hours Equivalent | EOE Average slowdown load/rate, % of PVT × i=1n(EOE slowdown hrsi) |  |  |
| EO Slowdown Load/rate Reduction as Shutdown Hours Equivalent | EO Average slowdown load/rate, % of PVT × i=1n(EO slowdown hrsi) |  |  |
| EOE Effective Operating Hours @ PVT | Total Available operating hours for the month-Shutdown/TA hours-EOE Slowdown load reduction as Shutdown hours Equivalentother reasons+EOE Slowdown load reduction as Shutdown hours Equivalent(Rampup/RampDown) |  |  |
| EO Effective Operating Hours @ PVT | Total Available operating hours for the month-Shutdown/TA hours-EO Slowdown load reduction as Shutdown hours Equivalentother reasons+EO Slowdown load reduction as Shutdown hours Equivalent(Rampup/RampDown) |  |  |

Table 4 Maintenance Details

The screen below depicts the Maintenance Details Table, which is captured in the digitized platform.

NOTE: The below screen is read-only. The users cannot change the values.

Figure 9 AOP Maintenance Details Table

#### Calendar Year Based Production Calculations Details/Production AOP

Using the Business Demand, Production Volume data and Maintenance Plan (Operating Hours), the month-wise production quantity is derived.

The month-wise budget production for each product is calculated in MT and KT, the formula for which is mentioned below.

Production AOP (MT) =(Operating hours in a month (hrs)  * Percentage of product demand * MC Value of product(TPH)) / 100

Operating hours for MEG = Total Available hours of the month – Shutdown/TA hours of the month

Operating hours for EOE = Total Available operating hours for the month - Shutdown/TA hours - EOE Slowdown load reduction as Shutdown hours Equivalent (other reasons) + EOE Slowdown load reduction as Shutdown hours Equivalent (Ramp up/Ramp Down)

Operating hours for EO = Total Available operating hours for the month - Shutdown/TA hours-EO Slowdown load reduction as Shutdown hours Equivalent (other reasons) + EO Slowdown load reduction as Shutdown hours Equivalent (Ramp up/Ramp Down)

Hence, month–wise production is,

Total Budgeted Production  = ∑All the values for the month

The month-wise budget production is further used to derive the annual plan in KT, and this is part of the final report.

This information is captured in the AOP digitization portal as seen in the screenshot below.

Figure 10 Digitized AOP screen for Production AOP month wise

The ‘Calculate’ button on this screen allows users to recalculate Production values when changes are made to configuration settings (e.g., MEG yield), business demand, or when line items are added to slowdown and shutdown activities.

### Norms

Below are the steps captured to derive the Norms value.

#### Normal Operation Norms

All the specific consumption norms are set with One-unit EOE as the basis.

For calculation purpose in the Digitized AOP platform, based on the DCT provided by the user, Norms are broadly classified as –

- Historical Best

- Based on Calculations

- Manual Entries

##### 3.1.2.1.1 Historical Best

Based on the historical data of last 5 years, corresponding to the MCU production, the best-achieved feed slate period is filtered. This data is further used for the calculation of consumption norms, which is the average of the norms during that period, excluding the identified outliers.

Below is the logic formulated to derive the Historical best Normal Operation Norms –

Figure 11 Digitized AOP Steps to derive month wise Historical Best Norms

The user has provision to overwrite/update the value by providing a valid reason for the historical best particulars. This overwritten value, if any, will be highlighted in RED color.

The list of particulars to be derived using the Historical Best logic is specified in the DCT provided by the user. These particulars may vary from plant to plant.

##### 3.1.2.1.2 Based on Calculations

For the MEG vertical, particulars like ethylene, oxygen, HP Steam, BFW water etc. are formulated based on the stoichiometric calculations provided by the user in the DCT.

The formula can vary from plant to plant.

E.g. the Raw Material norms for ethylene and oxygen for HMD MEG1 are derived using the calculations mentioned in the below table.

| Plant | Category | Particular | Calculation |
| --- | --- | --- | --- |
| HMD MEG1 | Raw Material | Ethylene | (((EOE Losses for ethylene*28*100/44/{selectivity})+([METHANE Norms]*28*0.28/16/0.54))/Ethylene Purity)+(0.135/24/([EOE]/[EOE Op hrs])) |
|  |  | Oxygen | (EOE Losses for oxygen*(100*2.1818/{selectivity}-1.818)+([METHANE Norms]*32*0.05/16/0.54))/Oxygen Purity |

Table 5 Raw Material Norms calculations

The values for losses and purity can be captured from the Lab analysis or Past/last year's operations. The user has the provision to enter the value manually in the Configuration screen.

Selectivity is defined as the efficiency of the catalyst. The data for selectivity will be extrapolated based on the previous year’s data till the end of the catalyst run (EOR). The user will have an additional capability to insert the data whenever the catalyst changeover takes place. This is a need as there is no previous data for the catalyst selectivity during the start of the run (SOR).

The image below is for reference.

Figure 12 Example of selectivity Extrapolation

Selectivity, Losses and Purity can be modified/added by the user in the Configuration Screen under the Norms Subsection.

Figure 13 Digitized AOP screen for Catalyst Selectivity month wise

In the Normal Operations Norms screen the cells for particulars with formula, e.g. HP Steam Dis, will be read only. If the values for such particulars are to be modified then the respective inputs should be modified, e.g. Total HP Steam consumption in the Configuration Screen.

##### 3.1.2.1.3 Manual Entries

The particulars under the Chemicals and Catalyst sections are manually entered by the user, who must also provide the corresponding norm values in the Normal Operation Norms screen.

The screenshot below highlights the details captured in the digitized AOP platform in the Normal Operations Norms screen.

Figure 14 Digitized AOP screen for Normal operations Norms

The Normal Operation Normas screen also captures the Intermediate calculations required for Norms calculation as shown in the below screen shot –

The ‘Calculate’ button on the screen will enable if the user makes any change in the dependent inputs in the Configuration screen or overwrite the norms values like Methane used in other norms calculations.

The ‘Save’ button on the above screen enables the user to save any norm value updated manually for historical best norms or any calculated value.

#### Shutdown Norms

The Shutdown norms are recorded for the months in which shutdown activities are planned, as specified in the Shutdown Activities screen.

The shutdown quantity utilization data includes TA, asset maintenance linked shutdown/ slowdown activities, etc..

The use must enter the consumption data (absolute values) for each line item.

The below screenshot depicts the Shutdown Norms captured in the AOP digitized platform.

Figure 15 Digitized AOP screen for Shutdown Norms

#### Slowdown Norms

Slowdown norms are recorded for the months in which slowdown activities are planned, as specified in the Slowdown Activities screen.

The values for slowdown norms are same as the Normal Operations norms for the month.

The below screenshot depicts the Shutdown Norms captured in the AOP digitized platform.

Figure 16 Digitized AOP screen for Shutdown Norms

#### Budget Norms/Consumption AOP

Using the values of Normal operations Norms, Shutdown Consumptions and Slowdown Norms, the final Consumption AOP value is calculated as below –

Consumption AOP for months without shutdown or slowdown = Normal Operations Norms

Consumption AOP for months with shutdown or slowdown = Total Consumption/Total production

Where,

Total Consumption = (Normal Operation Norms*EOE MC value*EOE operating hours)+i=1n(Slowdown Norms*EOE slowdown hours*EOE slowdown rate)n+Shutdown Consumption

Total Production = (EOE MC value*EOE operating hours)+i=1n(EOE slowdown hours*EOE slowdown rate)n

The below screenshot depicts the Shutdown Norms captured in the AOP digitized platform.

Figure 17 Digitized AOP screen for Budget Norms or Consumption Norms

### Feedstock Availability

Availability of Feedstock/Raw Material defines maximum attainable production in a plant.

In the case of MEG technology, the availability of Ethylene and Oxygen will be accounted for planning of unit production.

For the Rev 0 for AOP production/norms will consider the 100% availability of the required.

Figure 18 Digitized AOP screen for Feed Stock Availability

- These quantities will be compared against feedstock requirements and used for optimizing the production to meet the constrained feedstock availability as per below mentioned logic.

Reduced No of Operating Hrs to Maximize the production by making Delta Feedstock (difference of Available and Required feedstock) = 0

### Other Cost (To generate cost sheet - Business)

- Process consumables will be discussed with RIL and TBD for a path forward.

- Maintenance consumables will be discussed with RIL and TBD for a path forward.

# AOP Solution Overview/Output

- Rev 0 - Budgeted Production and Norms.

- Excel Tables outputs – List and end-state views are required.

- Final format of AOP budget to be uploaded in BPC.

- Plant-wise Basis report for APEX review.

- Annual AOP Cost Report.

- Plant Production Summary.

- Month wise Production Plan.

- Month wise Raw data.

- Turnaround Report.

- Annual Production Plan.

- Planned Contribution.

# Key Assumptions and Exemptions

The above scope of work should be read with the following assumptions made

- Data Acceptance

- DCT provided by RIL team